###### SPTECH Silicon NPN Power Transistor         2SC4237

###### DESCRIPTION

·Collector-Emitter Sustaining Voltage-

: VCEO(SUS)= 800V(Min)

·Fast Switching speed

APPLICATIONS

·Color TV horizontal output applications

### ABSOLUTE MAXIMUM RATINGS(Ta=25℃)

| SYMBOL | PARAMETER | VALUE | UNIT |
| --- | --- | --- | --- |
| VCBO | Collector-Base Voltage | 1200 | V |
| VCEO | Collector-Emitter Voltage | 800 | V |
| VEBO | Emitter-Base Voltage | 7 | V |
| IC | Collector Current-Continuous | 12 | A |
| ICM | Collector Current-Peak | 25 | A |
| IB | Base Current-Continuous | 4 | A |
| IBM | Base Current-Peak | 8 | A |
| PT | Total Power Dissipation @ TC=25℃ | 150 | W |
| TJ | Junction Temperature | 150 | ℃ |
| Tstg | Storage Temperature Range | -55~150 | ℃ |

THERMAL CHARACTERISTICS

| SYMBOL | PARAMETER | MAX | UNIT |
| --- | --- | --- | --- |
| Rth j-c | Thermal Resistance,Junction to Case | 0.83 | ℃/W |

###### SPTECH Silicon NPN Power Transistor         2SC4237

ELECTRICAL CHARACTERISTICS

TC=25℃ unless otherwise specified

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP. | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| VCEO(SUS) | Collector-Emitter Sustaining Voltage | IC= 10mA; IB= 0 | 800 |  |  | V |
| VCE(sat) | Collector-Emitter Saturation Voltage | IC= 5A; IB= 1A |  |  | 1.0 | V |
| VBE(sat) | Base-Emitter Saturation Voltage | IC= 5A; IB= 1A |  |  | 1.5 | V |
| ICBO | Collector Cutoff Current | At rated Voltage |  |  | 100 | μA |
| ICEO | Collector Cutoff Current | At rated Voltage |  |  | 100 | μA |
| IEBO | Emitter Cutoff Current | At rated Voltage |  |  | 100 | μA |
| hFE-1 | DC Current Gain | IC= 5A ; VCE= 5V | 18 |  | 28 |  |
| hFE-2 | DC Current Gain | IC= 1mA ; VCE= 5V | 5 |  |  |  |
| fT | Current-Gain—Bandwidth Product | IC= 1A ; VCE= 10V |  | 8 |  | MHz |
| Switching times |  |  |  |  |  |  |
| ton | Turn-on Time | IC= 5A , IB1= 1A; IB2= -2A RL= 50Ω; VBB2= 4V |  |  | 0.5 | μs |
| tstg | Storage Time |  |  |  | 3.5 | μs |
| tf | Fall Time |  |  |  | 0.3 | μs |