###### SPTECH Silicon NPN Power Transistor         2SC3058

###### DESCRIPTION

·High Collector-Emitter Sustaining Voltage-

: VCEO(sus)= 400V(Min)

·High Switching Speed

·Wide Area of Safe Operation

APPLICATIONS

·Switching regulators

·Motor controls

·Ultrasonic generators

·Class C and D amplifiers

·Deflection circuits

### ABSOLUTE MAXIMUM RATINGS(Ta=25℃)

| SYMBOL | PARAMETER | VALUE | UNIT |
| --- | --- | --- | --- |
| VCBO | Collector-Base Voltage | 600 | V |
| VCEO | Collector-Emitter Voltage | 400 | V |
| VEBO | Emitter-Base voltage | 7 | V |
| IC | Collector Current-Continuous | 30 | A |
| ICM | Collector Current-Peak | 50 | A |
| IB | Base Current-Continuous | 10 | A |
| PC | Collector Power Dissipation @ TC=25℃ | 200 | W |
| TJ | Junction Temperature | 175 | ℃ |
| Tstg | Storage Temperature Range | -65~175 | ℃ |

###### SPTECH Silicon NPN Power Transistor         2SC3058

ELECTRICAL CHARACTERISTICS

TC=25℃ unless otherwise specified

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP. | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| VCEO(SUS) | Collector-Emitter Sustaining Voltage | IC= 30mA; RBE= ∞ | 400 |  |  | V |
| V(BR)CBO | Collector-Base Breakdown Voltage | IC= 1 mA; IE= 0 | 600 |  |  | V |
| V(BR)EBO | Emitter-Base Breakdown Voltage | IE= 1mA; IC= 0 | 7 |  |  | V |
| VCE(sat) | Collector-Emitter Saturation Voltage | IC= 20A; IB= 4A |  |  | 1.0 | V |
| VBE(sat) | Base-Emitter Saturation Voltage | IC= 20A; IB= 4A |  |  | 1.5 | V |
| ICBO | Collector Cutoff Current | VCB= 500V; IE= 0 VCB= 500V; IE= 0; TC= 100℃ |  |  | 0.1 2.0 | mA |
| IEBO | Emitter Cutoff Current | VEB= 6V; IC= 0 |  |  | 100 | μA |
| hFE | DC Current Gain | IC= 20A; VCE= 5V | 10 |  | 40 |  |
| fT | Current-Gain—Bandwidth Product | IC= 4A; VCE= 10V | 15 |  |  | MHz |
| COB | Output Capacitance | IE= 0; VCB= 10V; ftest= 1.0MHz |  | 420 |  | pF |
| Switching times |  |  |  |  |  |  |
| tr | Rise Time | IC= 20A, IB1= -IB2= 4A; VCC= 150V |  |  | 0.5 | μs |
| tstg | Storage Time |  |  |  | 3.0 | μs |
| tf | Fall Time |  |  |  | 0.3 | μs |