###### SPTECH Silicon PNP Power Transistor             2SA1744

###### DESCRIPTION

·Collector-Emitter Sustaining Voltage-

: VCEO(SUS)= -60V(Min)

·High DC Current Gain-

: hFE= 100(Min)@ (VCE= -2V , IC= -3A)

·Low Saturation Voltage-

: VCE(sat)= -0.3V(Max)@ (IC= -8A, IB= -0.4A)

APPLICATIONS

·This type of power transistor is developed for high-speed

switching and features a high hFE at low VCE(sat),which is

ideal for use as a driver in DC/DC converters and actuators.

ABSOLUTE MAXIMUM RATINGS(Ta=25℃)

| SYMBOL | PARAMETER | VALUE | UNIT |
| --- | --- | --- | --- |
| VCBO | Collector-Base Voltage | -100 | V |
| VCEO | Collector-Emitter Voltage | -60 | V |
| VEBO | Emitter-Base Voltage | -7.0 | V |
| IC | Collector Current-Continuous | -15 | A |
| ICM | Collector Current-Pulse | -30 | A |
| IB | Base Current-Continuous | -7.5 | A |
| PT | Total Power Dissipation @TC=25℃ | 30 | W |
|  | Total Power Dissipation @Ta=25℃ | 2.0 |  |
| TJ | Junction Temperature | 150 | ℃ |
| Tstg | Storage Temperature | -55~150 | ℃ |

###### SPTECH Silicon PNP Power Transistor         2SA1744

ELECTRICAL CHARACTERISTICS

Tj=25℃ unless otherwise specified

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP. | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| VCEO(SUS) | Collector-Emitter Sustaining Voltage | IC= -8.0A ; IB= -0.8A, L= 1mH | -60 |  |  | V |
| VCEX(SUS) | Collector-Emitter Sustaining Voltage | IC= -8.0A ; IB1=-IB2= -0.8A, VBE(OFF)=1.5V, L=180μH,clamped | -60 |  |  | V |
| VCE(sat)-1 | Collector-Emitter Saturation Voltage | IC= -8A; IB= -0.4A |  |  | -0.3 | V |
| VCE(sat)-2 | Collector-Emitter Saturation Voltage | IC= -12A; IB= -0.6A |  |  | -0.5 | V |
| VBE(sat)-1 | Base-Emitter Saturation Voltage | IC= -8A; IB= -0.4A |  |  | -1.2 | V |
| VBE(sat)-2 | Base-Emitter Saturation Voltage | IC= -12A; IB= -0.6A |  |  | -1.5 | V |
| ICBO | Collector Cutoff Current | VCB= -60V ; IE=0 |  |  | -10 | μA |
| ICER | Collector Cutoff Current | VCE= -60V ; RBE= 50Ω,Ta=125℃ |  |  | -1.0 | mA |
| ICEX | Collector Cutoff Current | VCE= -60V; VBE(off)= -1.5V VCE= -60V; VBE(off)= -1.5V,Ta=125℃ |  |  | -10 -1.0 | μA mA |
| IEBO | Emitter Cutoff Current | VEB= -5V; IC=0 |  |  | -10 | μA |
| hFE-1 | DC Current Gain | IC= -1.5A; VCE= -2V | 100 |  |  |  |
| hFE-2 | DC Current Gain | IC= -3.0A; VCE= -2V | 100 |  | 400 |  |
| hFE-3 | DC Current Gain | IC= -8.0A; VCE= -2V | 60 |  |  |  |
| COB | Output Capacitance | IE=0; VCB= -10V; f= 1.0MHz |  | 300 |  | pF |
| fT | Current-Gain—Bandwidth Product | IC= -1.5A; VCE= -10V |  | 80 |  | MHz |
| Switching times |  |  |  |  |  |  |
| ton | Turn-on Time | IC= -8.0A, RL= 6.3Ω, IB1= -IB2= -0.4A, VCC≈ -50V |  |  | 0.3 | μs |
| tstg | Storage Time |  |  |  | 1.5 | μs |
| tf | Fall Time |  |  |  | 0.3 | μs |

- hFE-2 Classifications

| M | L | K |
| --- | --- | --- |
| 100-200 | 150-300 | 200-400 |