###### isc Silicon NPN Power Transistor                 2SC4550

###### DESCRIPTION

·Collector-Emitter Sustaining Voltage-

: VCEO(SUS)= 60V(Min)

·High DC Current Gain-

: hFE= 100(Min)@ (VCE= 2V , IC= 1.5A)

·Low Saturation Voltage-

: VCE(sat)= 0.3V(Max)@ (IC= 4A, IB= 0.2A)

APPLICATIONS

·Designed for use as a driver in DC/DC converters and

actuators.

ABSOLUTE MAXIMUM RATINGS(Ta=25℃)

| SYMBOL | PARAMETER | VALUE | UNIT |
| --- | --- | --- | --- |
| VCBO | Collector-Base Voltage | 100 | V |
| VCEO | Collector-Emitter Voltage | 60 | V |
| VEBO | Emitter-Base Voltage | 7.0 | V |
| IC | Collector Current-Continuous | 7.0 | A |
| ICM | Collector Current-Pulse | 14 | A |
| IB | Base Current-Continuous | 3.5 | A |
| PT | Total Power Dissipation @TC=25℃ | 30 | W |
|  | Total Power Dissipation @Ta=25℃ | 2.0 |  |
| TJ | Junction Temperature | 150 | ℃ |
| Tstg | Storage Temperature | -55~150 | ℃ |

###### SPTECH Silicon NPN Power Transistor         2SC4550

ELECTRICAL CHARACTERISTICS

Tj=25℃ unless otherwise specified

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP. | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| VCEO(SUS) | Collector-Emitter Sustaining Voltage | IC=50mA,Ib=0 | 60 |  |  | V |
| VCE(sat)-1 | Collector-Emitter Saturation Voltage | IC= 4A; IB= 0.2A |  |  | 0.3 | V |
| VCE(sat)-2 | Collector-Emitter Saturation Voltage | IC= 6A; IB= 0.3A |  |  | 0.5 | V |
| VBE(sat)-1 | Base-Emitter Saturation Voltage | IC= 4A; IB= 0.2A |  |  | 1.2 | V |
| VBE(sat)-2 | Base-Emitter Saturation Voltage | IC= 6A; IB= 0.3A |  |  | 1.5 | V |
| ICBO | Collector Cutoff Current | VCB= 60V ; IE= 0 |  |  | 10 | μA |
| ICER | Collector Cutoff Current | VCE= 60V ; RBE= 50Ω,Ta=125℃ |  |  | 1.0 | mA |
| IEBO | Emitter Cutoff Current | VEB= 5V; IC= 0 |  |  | 10 | μA |
| hFE-1 | DC Current Gain | IC= 0.7A ; VCE= 2V | 100 |  |  |  |
| hFE-2 | DC Current Gain | IC= 1.5A ; VCE= 2V | 100 |  | 400 |  |
| hFE-3 | DC Current Gain | IC= 4.0A ; VCE= 2V | 60 |  |  |  |
| COB | Output Capacitance | IE= 0 ; VCB= 10V; f= 1.0MHz |  | 100 |  | pF |
| fT | Current-Gain—Bandwidth Product | IC= 1A ; VCE= 10V | 30 |  |  | MHz |
| Switching times |  |  |  |  |  |  |
| ton | Turn-on Time | IC= 4.0A ,RL= 12.5Ω, IB1= -IB2= 0.2A,VCC≈ 50V |  |  | 0.3 | μs |
| tstg | Storage Time |  |  |  | 1.5 | μs |
| tf | Fall Time |  |  |  | 0.3 | μs |

- hFE-2 Classifications

| M | L | K |
| --- | --- | --- |
| 100-200 | 150-300 | 200-400 |