###### SPTECH Silicon NPN Power Transistor         2SC4110

###### DESCRIPTION

·High Collector-Emitter Breakdown Voltage-

: V(BR)CEO= 400V(Min)

·High Switching Speed

·Wide Area of Safe Operation

APPLICATIONS

·Designed for switching regulator and general purpose

applications.

### ABSOLUTE MAXIMUM RATINGS(Ta=25℃)

| SYMBOL | PARAMETER | VALUE | UNIT |
| --- | --- | --- | --- |
| VCBO | Collector-Base Voltage | 500 | V |
| VCEO | Collector-Emitter Voltage | 400 | V |
| VEBO | Emitter-Base voltage | 7 | V |
| IC | Collector Current-Continuous | 25 | A |
| ICM | Collector Current-Peak PW300s, duty cycle10% | 40 | A |
| IB | Base Current-Continuous | 8 | A |
| PC | Collector Power Dissipation @ TC=25℃ | 160 | W |
|  | Collector Power Dissipation @ Ta=25℃ | 2.5 |  |
| TJ | Junction Temperature | 150 | ℃ |
| Tstg | Storage Temperature Range | -55~150 | ℃ |

###### SPTECH Silicon NPN Power Transistor         2SC4110

ELECTRICAL CHARACTERISTICS

TC=25℃ unless otherwise specified

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP. | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| V(BR)CBO | Collector-Base Breakdown Voltage | IC= 1mA; IE= 0 | 500 |  |  | V |
| V(BR)CEO | Collector-Emitter Breakdown Voltage | IC= 10mA; RBE= ∞ | 400 |  |  | V |
| V(BR)EBO | Emitter-Base Breakdown Voltage | IE= 1m A; IC= 0 | 7 |  |  | V |
| VCE(sat) | Collector-Emitter Saturation Voltage | IC= 16A; IB= 3.2A |  |  | 0.8 | V |
| VBE(sat) | Base-Emitter Saturation Voltage | IC= 16A; IB= 3.2A |  |  | 1.5 | V |
| ICBO | Collector Cutoff Current | VCB= 400V ; IE= 0 |  |  | 10 | μA |
| IEBO | Emitter Cutoff Current | VEB= 5V; IC= 0 |  |  | 10 | μA |
| hFE-1 | DC Current Gain | IC= 3.2A ; VCE= 5V | 15 |  | 50 |  |
| hFE-2 | DC Current Gain | IC= 16A ; VCE= 5V | 10 |  |  |  |
| hFE-3 | DC Current Gain | IC= 10mA ; VCE= 5V | 10 |  |  |  |
| fT | Current-Gain—Bandwidth Product | IC= 3.2A ; VCE= 10V |  | 20 |  | MHz |
| COB | Output Capacitance | IE= 0; VCB= 10V; ftest= 1.0MHz |  | 300 |  | pF |
| Switching Times |  |  |  |  |  |  |
| ton | Turn-on Time | IC= 20A, IB1= 4A; IB2= -8A RL= 10Ω; VCC= 200V |  |  | 0.5 | μs |
| tstg | Storage Time |  |  |  | 2.5 | μs |
| tf | Fall Time |  |  |  | 0.3 | μs |

- hFE-1 Classifications

| L | M | N |
| --- | --- | --- |
| 15-30 | 20-40 | 30-50 |