###### SPTECH Silicon NPN Power Transistor         2SC3907

###### DESCRIPTION

·Collector-Emitter Breakdown Voltage-

: V(BR)CEO= 180V(Min)

·Good Linearity of hFE

·Complement to Type 2SA1516

APPLICATIONS

·Power amplifier applications

·Recommend for 80W high fidelity audio frequency

amplifier output stage applications

### ABSOLUTE MAXIMUM RATINGS(Ta=25℃)

| SYMBOL | PARAMETER | VALUE | UNIT |
| --- | --- | --- | --- |
| VCBO | Collector-Base Voltage | 180 | V |
| VCEO | Collector-Emitter Voltage | 180 | V |
| VEBO | Emitter-Base Voltage | 5 | V |
| IC | Collector Current-Continuous | 12 | A |
| IB | Base Current-Continuous | 1.2 | A |
| PC | Collector Power Dissipation @ TC=25℃ | 130 | W |
| TJ | Junction Temperature | 150 | ℃ |
| Tstg | Storage Temperature Range | -55~150 | ℃ |

###### SPTECH Silicon NPN Power Transistor         2SC3907

ELECTRICAL CHARACTERISTICS

TC=25℃ unless otherwise specified

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP. | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| V(BR)CEO | Collector-Emitter Breakdown Voltage | IC= 50mA; IB= 0 | 180 |  |  | V |
| VCE(sat) | Collector-Emitter Saturation Voltage | IC= 8A; IB= 0.8A |  |  | 2.0 | V |
| VBE(on) | Base-Emitter On Voltage | IC= 7A ; VCE= 5V |  |  | 1.5 | V |
| ICBO | Collector Cutoff Current | VCB= 180V; IE= 0 |  |  | 5 | μA |
| IEBO | Emitter Cutoff Current | VEB= 5V; IC= 0 |  |  | 5 | μA |
| hFE-1 | DC Current Gain | IC= 1A; VCE= 5V | 55 |  | 180 |  |
| hFE-2 | DC Current Gain | IC= 7A; VCE= 5V | 35 |  |  |  |
| COB | Output Capacitance | IE=0; VCB= 10V;ftest= 1.0MHz |  | 270 |  | pF |
| fT | Current-Gain—Bandwidth Product | IC= 1A; VCE= 5V |  | 30 |  | MHz |

- hFE-1 Classifications

| R | O |
| --- | --- |
| 55-110 | 90-180 |