###### SPTECH Silicon NPN Power Transistor          BU941P

###### DESCRIPTION

·High Voltage

·DARLINGTON

APPLICATIONS

·High ruggedness electronic ignitions

·High voltage ignition coil driver

### Absolute maximum ratings (TBaB=25℃)

| SYMBOL | PARAMETER | VALUE | UNIT |
| --- | --- | --- | --- |
| VBCBOB | Collector-Base Voltage | 500 | V |
| VBCEOB | Collector-Emitter Voltage | 400 | V |
| VBEBOB | Emitter-Base Voltage | 5 | V |
| IBCB | Collector Current- Continuous | 15 | A |
| IBCMB | Collector Current-Peak | 30 | A |
| IBBB | Base Current | 1 | A |
| IBBMB | Base Current-Peak | 5 | A |
| PBCB | Collector Power Dissipation @TBCB=25℃ | 155 | W |
| TBjB | Junction Temperature | 150 | ℃ |
| TBstgB | Storage Temperature Range | -65~150 | ℃ |

THERMAL CHARACTERISTICS

| SYMBOL | PARAMETER | MAX | UNIT |
| --- | --- | --- | --- |
| RBth j-cB | Thermal Resistance, Junction to Case | 0.97 | ℃/W |

###### SPTECH Silicon NPN Power Transistor          BU941P

ELECTRICAL CHARACTERISTICS

TBCB=25℃ unless otherwise specified

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| VBCEO(SUS)B | Collector-Emitter Sustaining Voltage | IBCB=50mA; IBBB= 0 | 400 |  |  | V |
| VBCE(sat)-1B | Collector-Emitter Saturation Voltage | IBCB= 8 A; IBBB= 100mA |  |  | 1.6 | V |
| VBCE(sat)-2B | Collector-Emitter Saturation Voltage | IBCB= 10 A; IBBB= 250mA |  |  | 1.8 | V |
| VBCE(sat)-3B | Collector-Emitter Saturation Voltage | IBCB= 12 A; IBBB= 300mA |  |  | 2.0 | V |
| VBBE(sat)-1B | Base-Emitter Saturation Voltage | IBCB= 8 A; IBBB= 100mA |  |  | 2.2 | V |
| VBBE(sat)-2B | Base-Emitter Saturation Voltage | IBCB= 10 A; IBBB= 250mA |  |  | 2.5 | V |
| VBBE(sat)-3B | Base-Emitter Saturation Voltage | IBCB= 12 A; IBBB= 300mA |  |  | 2.7 | V |
| IBCESB | Collector Cutoff Current | VBCEB= 500V;VBBEB= 0 VBCEB= 500V;VBBEB= 0;TBjB= 125℃ |  |  | 0.1 0.5 | mA |
| IBCEOB | Collector Cutoff Current | VBCEB= 450V; IBBB= 0 VBCEB= 450V; IBBB= 0;TBjB= 125℃ |  |  | 0.1 0.5 | mA |
| IBEBOB | Emitter Cutoff Current | VBEBB= 5V; IBCB= 0 |  |  | 20 | mA |
| hBFEB | DC Current Gain | IBCB= 5A ; VBCEB= 10V | 300 |  |  |  |
| VBECFB | C-E Diode Forward Voltage | IBFB= 10A |  |  | 2.5 | V |