###### SPTECH Silicon NPN Darlington Power Transistor   2SD1525

DESCRIPTION

·High DC Current Gain

: hFE= 1000(Min.)@ IC= 20A

·Collector-Emitter Breakdown Voltage-

: V(BR)CEO = 100V(Min.)

APPLICATIONS

·Designed for high current switching applications.

ABSOLUTE MAXIMUM RATINGS(Ta=25℃)

| SYMBOL | PARAMETER | VALUE | UNIT |
| --- | --- | --- | --- |
| VCBO | Collector-Base Voltage | 100 | V |
| VCEO | Collector-Emitter Voltage | 100 | V |
| VEBO | Emitter-Base Voltage | 5 | V |
| IC | Collector Current-Continuous | 30 | A |
| IB | Base Current- Continuous | 5 | A |
| PC | Collector Power Dissipation @TC=25℃ | 150 | W |
| Tj | Junction Temperature | 150 | ℃ |
| Tstg | Storage Temperature Range | -55~150 | ℃ |

###### SPTECH Silicon NPN Darlington Power Transistor   2SD1525

ELECTRICAL CHARACTERISTICS

TC=25℃ unless otherwise specified

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP. | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| V(BR)CEO | Collector-Emitter Breakdown Voltage | IC= 30mA, IB= 0 | 100 |  |  | V |
| VCE(sat) | Collector-Emitter Saturation Voltage | IC= 20A, IB= 0.2A |  |  | 1.5 | V |
| VBE(sat) | Base-Emitter Saturation Voltage | IC= 20A, IB= 0.2A |  |  | 2.5 | V |
| ICBO | Collector Cutoff current | VCB= 100V, IE= 0 |  |  | 0.1 | mA |
| IEBO | Emitter Cutoff current | VEB= 5V, IC= 0 |  |  | 10 | mA |
| hFE-1 | DC Current Gain | IC= 20A; VCE= 5V | 1000 |  |  |  |
| hFE-2 | DC Current Gain | IC= 30A; VCE= 5V | 200 |  |  |  |
| VECF | C-E Diode Forward Voltage | IF= 10A |  |  | 3.0 | V |
| fT | Current-Gain—Bandwidth Product | IC= 1A; VCE= 5V |  | 10 |  | MHz |
| COB | Output Capacitance | IE= 0; VCB= 10V; ftest= 1MHz |  | 500 |  | pF |
| Switching Times |  |  |  |  |  |  |
| ton | Turn-On Time | IB1 = IB2= 10mA; VCC= 50V; RL= 10Ω |  | 1.5 |  | μs |
| tstg | Storage Time |  |  | 10 |  | μs |
| tf | Fall Time |  |  | 1.5 |  | μs |