###### SPTECH Silicon NPN Darlington Power Transistor   MJ11012

###### DESCRIPTION

·Collector-Emitter Breakdown Voltage

: V(BR)CEO= 60V(Min.)

·High DC Current Gain-

: hFE= 1000(Min.)@IC= 20A

·Low Collector Saturation Voltage-

: VCE (sat)= 3.0V(Max.)@ IC= 20A

·Complement to the PNP MJ11011

APPLICATIONS

·Designed for use as output devices in complementary

general purpose amplifier applications.

### Absolute maximum ratings (Ta=25℃)

| SYMBOL | PARAMETER | VALUE | UNIT |
| --- | --- | --- | --- |
| VCBO | Collector-Base Voltage | 60 | V |
| VCEO | Collector-Emitter Voltage | 60 | V |
| VEBO | Emitter-Base Voltage | 5 | V |
| IC | Collector Current-Continunous | 30 | A |
| ICM | Collector Current-Peak | 50 | A |
| IB | Base Current-Continunous | 1 | A |
| PC | Collector Power Dissipation @TC=25℃ | 200 | W |
| Tj | Junction Temperature | 200 | ℃ |
| Tstg | Storage Temperature Range | -55~+200 | ℃ |

THERMAL CHARACTERISTICS

| SYMBOL | PARAMETER | MAX | UNIT |
| --- | --- | --- | --- |
| Rth j-c | Thermal Resistance, Junction to Case | 0.87 | ℃/W |

###### SPTECH Silicon NPN Darlington Power Transistor   MJ11012

ELECTRICAL CHARACTERISTICS

TC=25℃ unless otherwise specified

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP. | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| V(BR)CEO | Collector-Emitter Breakdown Voltage | IC= 50mA; IB= 0 | 60 |  |  | V |
| VCE(sat)-1 | Collector-Emitter Saturation Voltage | IC= 20A; IB= 0.2A |  |  | 3.0 | V |
| V CE(sat)-2 | Collector-Emitter Saturation Voltage | IC= 30A; IB= 0.3A |  |  | 4.0 | V |
| VBE(sat)-1 | Base-Emitter Saturation Voltage | IC= 20A; IB= 0.2A |  |  | 3.5 | V |
| VBE(sat)-2 | Base-Emitter Saturation Voltage | IC= 30A; IB= 0.3A |  |  | 5.0 | V |
| ICBO | Collector Cutoff Current | VCB=60V; IE=0 VCB=60V; IE=0; TC=150℃ |  |  | 1.0 5.0 | mA |
| ICEO | Collector Cutoff Current | VCE= 60V; IB= 0 |  |  | 1.0 | mA |
| IEBO | Emitter Cutoff Current | VEB= 5V; IC= 0 |  |  | 5.0 | mA |
| hFE-1 | DC Current Gain | IC= 20A, VCE= 5V | 1000 |  |  |  |
| hFE-2 | DC Current Gain | IC= 30A, VCE= 5V | 200 |  |  |  |