###### SPTECH Silicon NPN Darlington Power Transistor   MJ11032

###### DESCRIPTION

·Collector-Emitter Breakdown Voltage

: V(BR)CEO= 120V(Min.)

·High DC Current Gain-

: hFE= 1000(Min.)@IC= 25A

: hFE= 400(Min.)@IC= 50A

·Complement to the PNP MJ11033

APPLICATIONS

·Designed for use as output devices in complementary

general purpose amplifier applications.

### Absolute maximum ratings (Ta=25℃)

| SYMBOL | PARAMETER | VALUE | UNIT |
| --- | --- | --- | --- |
| VCBO | Collector-Base Voltage | 120 | V |
| VCEO | Collector-Emitter Voltage | 120 | V |
| VEBO | Emitter-Base Voltage | 5 | V |
| IC | Collector Current-Continunous | 50 | A |
| ICM | Collector Current-Peak | 100 | A |
| IB | Base Current-Continunous | 2 | A |
| PC | Collector Power Dissipation @TC=25℃ | 300 | W |
| Tj | Junction Temperature | 200 | ℃ |
| Tstg | Storage Temperature Range | -55~+200 | ℃ |

THERMAL CHARACTERISTICS

| SYMBOL | PARAMETER | MAX | UNIT |
| --- | --- | --- | --- |
| Rth j-c | Thermal Resistance, Junction to Case | 0.584 | ℃/W |

###### SPTECH Silicon NPN Darlington Power Transistor   MJ11032

ELECTRICAL CHARACTERISTICS

TC=25℃ unless otherwise specified

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP. | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| V(BR)CEO | Collector-Emitter Breakdown Voltage | IC= 50mA; IB= 0 | 120 |  |  | V |
| VCE(sat)-1 | Collector-Emitter Saturation Voltage | IC= 25A; IB= 250mA |  |  | 2.5 | V |
| V CE(sat)-2 | Collector-Emitter Saturation Voltage | IC= 50A; IB= 500mA |  |  | 3.5 | V |
| VBE(sat)-1 | Base-Emitter Saturation Voltage | IC= 25A; IB= 250mA |  |  | 3.0 | V |
| VBE(sat)-2 | Base-Emitter Saturation Voltage | IC= 50A; IB= 500mA |  |  | 4.5 | V |
| ICBO | Collector Cutoff Current | VCB=120V;IE=0 VCB=120V; IE=0; TC=150℃ |  |  | 2.0 5.0 | mA |
| ICEO | Collector Cutoff Current | VCE= 120V; IB= 0 |  |  | 2.0 | mA |
| IEBO | Emitter Cutoff Current | VEB= 5V; IC= 0 |  |  | 5.0 | mA |
| hFE-1 | DC Current Gain | IC= 25A, VCE= 5V | 1000 |  | 18000 |  |
| hFE-2 | DC Current Gain | IC= 50A, VCE= 5V | 400 |  |  |  |