###### SPTECH Silicon NPN Power Transistor         2SC3320

###### DESCRIPTION

·High Collector-Emitter Breakdown Voltage-

: V(BR)CEO= 400V(Min)

·High Switching Speed

·High Reliability

APPLICATIONS

·Switching regulators

·Ultrasonic generators

·High frequency inverters

·General purpose power amplifiers

### ABSOLUTE MAXIMUM RATINGS(Ta=25℃)

| SYMBOL | PARAMETER | VALUE | UNIT |
| --- | --- | --- | --- |
| VCBO | Collector-Base Voltage | 500 | V |
| VCEO | Collector-Emitter Voltage | 400 | V |
| VCEO(SUS) | Collector-Emitter Voltage | 400 | V |
| VEBO | Emitter-Base Voltage | 7 | V |
| IC | Collector Current-Continuous | 15 | A |
| IB | Base Current-Continuous | 5 | A |
| PC | Collector Power Dissipation @ TC=25℃ | 80 | W |
| TJ | Junction Temperature | 150 | ℃ |
| Tstg | Storage Temperature Range | -65~150 | ℃ |

THERMAL CHARACTERISTICS

| SYMBOL | PARAMETER | MAX | UNIT |
| --- | --- | --- | --- |
| Rth j-c | Thermal Resistance,Junction to Case | 1.56 | ℃/W |

###### SPTECH Silicon NPN Power Transistor         2SC3320

ELECTRICAL CHARACTERISTICS

TC=25℃ unless otherwise specified

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP. | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| V(BR)CEO | Collector-Emitter Breakdown Voltage | IC= 10mA ; IB= 0 | 400 |  |  | V |
| VCEO(SUS) | Collector-Emitter Sustaining Voltage | IC= 50mA ; IB= 0 | 400 |  |  | V |
| V(BR)CBO | Collector-Base Breakdown Voltage | IC= 1mA ; IE= 0 | 500 |  |  | V |
| V(BR)EBO | Emitter-Base Breakdown Voltage | IE= 1mA ; IC= 0 | 7 |  |  | V |
| VCE(sat) | Collector-Emitter Saturation Voltage | IC= 6A; IB= 1.2A |  |  | 1.0 | V |
| VBE(sat) | Base-Emitter Saturation Voltage | IC= 6A; IB= 1.2A |  |  | 1.5 | V |
| ICBO | Collector Cutoff Current | VCB= 500V ; IE= 0 |  |  | 1.0 | mA |
| IEBO | Emitter Cutoff Current | VEB= 7V; IC= 0 |  |  | 1.0 | mA |
| hFE | DC Current Gain | IC= 6A ; VCE= 5V | 10 |  |  |  |
| Switching times |  |  |  |  |  |  |
| ton | Turn-on Time | IC= 7.5A , IB1= 1.5A; IB2= -3A RL= 20Ω;PW=20μs Duty≤2% |  |  | 0.5 | μs |
| tstg | Storage Time |  |  |  | 1.5 | μs |
| tf | Fall Time |  |  |  | 0.15 | μs |