###### SPTECH Silicon NPN Power Transistor         2SC3528

###### DESCRIPTION

·Low Collector Saturation Voltage

·High Collector Current

·Good Linearity of hFE

APPLICATIONS

·Designed for switching regulator and high voltage

switching applications.

### ABSOLUTE MAXIMUM RATINGS(Ta=25℃)

| SYMBOL | PARAMETER | VALUE | UNIT |
| --- | --- | --- | --- |
| VCBO | Collector-Base Voltage | 500 | V |
| VCEO | Collector-Emitter Voltage | 400 | V |
| VEBO | Emitter-Base voltage | 7 | V |
| IC | Collector Current-Continuous | 20 | A |
| IB | Base Current-Continuous | 6 | A |
| PC | Collector Power Dissipation @ TC=25℃ | 125 | W |
|  | Collector Power Dissipation @ Ta=25℃ | 3 |  |
| TJ | Junction Temperature | 150 | ℃ |
| Tstg | Storage Temperature Range | -55~150 | ℃ |

###### SPTECH Silicon NPN Power Transistor         2SC3528

ELECTRICAL CHARACTERISTICS

TC=25℃ unless otherwise specified

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP. | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| VCEO(SUS) | Collector-Emitter Sustaining Voltage | IC=20mA ; IB=0 | 400 |  |  | V |
| VCE(sat) | Collector-Emitter Saturation Voltage | IC= 10A; IB= 2A |  |  | 1.0 | V |
| VBE(sat) | Base-Emitter Saturation Voltage | IC= 10A; IB= 2A |  |  | 1.5 | V |
| ICBO | Collector Cutoff Current | VCB= 500V ; IE= 0 |  |  | 100 | μA |
| IEBO | Emitter Cutoff Current | VEB= 7V; IC= 0 |  |  | 100 | μA |
| hFE-1 | DC Current Gain | IC= 2A ; VCE= 5V | 15 |  |  |  |
| hFE-2 | DC Current Gain | IC= 10A ; VCE= 5V | 10 |  |  |  |
| fT | Current-Gain—Bandwidth Product | IC= 1A ; VCE= 10V; f= 1MHz |  | 15 |  | MHz |
| Switching times |  |  |  |  |  |  |
| ton | Turn-On Time | IC= 10A; IB1= 2.0A, IB2= -2.0A; VCC= 125V |  |  | 1.0 | μs |
| tstg | Storage Time |  |  |  | 2.5 | μs |
| tf | Fall Time |  |  |  | 1.0 | μs |