###### SPTECH Silicon NPN Power Transistor         2SC3866

###### DESCRIPTION

·High Breakdown Voltage

: V(BR)CBO= 900V(Min)

·High Switching Speed

·High Reliability

APPLICATIONS

·Switching regulators

·Ultrasonic generators

·High frequency inverters

·General purpose power amplifiers

### ABSOLUTE MAXIMUM RATINGS(Ta=25℃)

| SYMBOL | PARAMETER | VALUE | UNIT |
| --- | --- | --- | --- |
| VCBO | Collector-Base Voltage | 900 | V |
| VCEO | Collector-Emitter Voltage | 800 | V |
| VEBO | Emitter-Base Voltage | 10 | V |
| IC | Collector Current-Continuous | 3 | A |
| IB | Base Current-Continuous | 1 | A |
| PC | Collector Power Dissipation @ TC=25℃ | 40 | W |
| TJ | Junction Temperature | 150 | ℃ |
| Tstg | Storage Temperature Range | -55~150 | ℃ |

THERMAL CHARACTERISTICS

| SYMBOL | PARAMETER | MAX | UNIT |
| --- | --- | --- | --- |
| Rth j-c | Thermal Resistance,Junction to Case | 3.0 | ℃/W |

###### SPTECH Silicon NPN Power Transistor         2SC3866

ELECTRICAL CHARACTERISTICS

TC=25℃ unless otherwise specified

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP. | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| V(BR)CEO | Collector-Emitter Breakdown Voltage | IC= 10mA; IB= 0 | 800 |  |  | V |
| V(BR)CBO | Collector-Base Breakdown Voltage | IC= 1mA; IE= 0 | 900 |  |  | V |
| V(BR)EBO | Emitter-Base Breakdown Voltage | IE= 1mA; IC= 0 | 10 |  |  | V |
| VCE(sat) | Collector-Emitter Saturation Voltage | IC= 1A; IB= 0.2A |  |  | 1.0 | V |
| VBE(sat) | Base-Emitter Saturation Voltage | IC= 1A; IB= 0.2A |  |  | 1.5 | V |
| ICBO | Collector Cutoff Current | VCB= 900V; IE= 0 |  |  | 1.0 | mA |
| IEBO | Emitter Cutoff Current | VEB= 10V; IC= 0 |  |  | 1.0 | mA |
| hFE | DC Current Gain | IC= 1A ; VCE= 5V | 10 |  |  |  |
| Switching times |  |  |  |  |  |  |
| ton | Turn-on Time | IC= 2A , IB1= 0.4A; IB2= -0.8A RL=150Ω; PW=20μs; Duty≤2% |  |  | 1.0 | μs |
| tstg | Storage Time |  |  |  | 4.0 | μs |
| tf | Fall Time |  |  |  | 0.8 | μs |