###### SPTECH Silicon PNP Power Transistor         2SA1073

###### DESCRIPTION

·High Collector-Emitter Breakdown Voltage-

V(BR)CEO= -160V(Min)

·Fast Switching Speed

·Wide Area of Safe Operation

·Complement to Type 2SC2523

APPLICATIONS

·High frequency power amplifier

·Audio power amplifiers

·Switching regulators

·DC-DC converters

### ABSOLUTE MAXIMUM RATINGS(Ta=25℃)

| SYMBOL | PARAMETER | VALUE | UNIT |
| --- | --- | --- | --- |
| VCBO | Collector-Base Voltage | -160 | V |
| VCEO | Collector-Emitter Voltage | -160 | V |
| VEBO | Emitter-Base Voltage | -7 | V |
| IC | Collector Current-Continuous | -12 | A |
| PC | Collector Power Dissipation @ TC=25℃ | 120 | W |
| TJ | Junction Temperature | 150 | ℃ |
| Tstg | Storage Temperature Range | -65~150 | ℃ |

###### SPTECH Silicon PNP Power Transistor         2SA1073

ELECTRICAL CHARACTERISTICS

TC=25℃ unless otherwise specified

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP. | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| V(BR)CEO | Collector-Emitter Breakdown Voltage | IC= -1mA; RBE= ∞ | -160 |  |  | V |
| V(BR)CBO | Collector-Base Breakdown Voltage | IC= -50μA; IE= 0 | -160 |  |  | V |
| V(BR)EBO | Emitter-Base Breakdown Voltage | IE= -50μA ; IC= 0 | -7 |  |  | V |
| VCE(sat) | Collector-Emitter Saturation Voltage | IC= -5A; IB= -0.5A |  |  | -1.8 | V |
| VBE(on) | Base-Emitter On Voltage | IC= -5A; VCE= -5V |  |  | -1.7 | V |
| ICBO | Collector Cutoff Current | VCB= -160V ; IE= 0 |  |  | -50 | μA |
| ICEO | Collector Cutoff Current | VCE= -160V ;RBE= ∞ |  |  | -1 | mA |
| IEBO | Emitter Cutoff Current | VEB= -7V; IC= 0 |  |  | -50 | μA |
| hFE-1 | DC Current Gain | IC= -1A; VCE= -5V | 60 |  | 200 |  |
| hFE-2 | DC Current Gain | IC= -7A; VCE= -5V | 40 |  |  |  |
| COB | Output Capacitance | IE= 0; VCB= -10V; f= 1.0MHz |  | 300 |  | pF |
| fT | Current-Gain—Bandwidth Product | IC= -1A; VCE= -10V; f= 10MHz |  | 60 |  | MHz |
| Switching Times |  |  |  |  |  |  |
| tr | Rise Time | IC= -7.5A; IB1= -IB2= -0.75A; RL= 4Ω |  | 0.15 |  | μs |
| tstg | Storage Time |  |  | 0.5 |  | μs |
| tf | Fall Time |  |  | 0.11 |  | μs |