###### SPTECH Silicon NPN Power Transistor         2SD1047

###### DESCRIPTION

·Collector-Emitter Breakdown Voltage-

: V(BR)CEO= 140V(Min)

·Good Linearity of hFE

·High Current Capability

·Wide Area of Safe Operation

·Complement to Type 2SB817

APPLICATIONS

·Recommend for 60W audio frequency amplifier output

stage applications

### ABSOLUTE MAXIMUM RATINGS(Ta=25℃)

| SYMBOL | PARAMETER | VALUE | UNIT |
| --- | --- | --- | --- |
| VCBO | Collector-Base Voltage | 160 | V |
| VCEO | Collector-Emitter Voltage | 140 | V |
| VEBO | Emitter-Base Voltage | 6 | V |
| IC | Collector Current-Continuous | 12 | A |
| ICP | Collector Current-Pulse | 15 | A |
| PC | Collector Power Dissipation @ TC=25℃ | 100 | W |
| TJ | Junction Temperature | 150 | ℃ |
| Tstg | Storage Temperature Range | -40~150 | ℃ |

###### SPTECH Silicon NPN Power Transistor         2SD1047

ELECTRICAL CHARACTERISTICS

TC=25℃ unless otherwise specified

| SYMBOL | PARAMETER | CONDITIONS | MIN | TYP. | MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| V(BR)CEO | Collector-Emitter Breakdown Voltage | IC= 30mA ; RBE= ∞ | 140 |  |  | V |
| V(BR)CBO | Collector-Base Breakdown Voltage | IC=1mA; IE= 0 | 160 |  |  | V |
| V(BR)EBO | Emitter-Base Breakdown Voltage | IE= 5mA; IC= 0 | 6 |  |  | V |
| VCE(sat) | Collector-Emitter Saturation Voltage | IC= 5.0A; IB= 0.5A |  | 0.6 | 2.5 | V |
| VBE(on) | Base -Emitter On Voltage | IC= 1A ; VCE= 5V |  |  | 1.5 | V |
| ICBO | Collector Cutoff Current | VCB= 80V ; IE= 0 |  |  | 100 | μA |
| IEBO | Emitter Cutoff Current | VEB= 4V; IC= 0 |  |  | 100 | μA |
| hFE-1 | DC Current Gain | IC= 1A ; VCE= 5V | 60 |  | 200 |  |
| hFE-2 | DC Current Gain | IC= 6A ; VCE= 5V | 20 |  |  |  |
| COB | Output Capacitance | IE= 0;VCB= 10V;ftest= 1.0MHz |  | 210 |  | pF |
| fT | Current-Gain—Bandwidth Product | IC= 1A ; VCE= 5V |  | 15 |  | MHz |
| Switching times |  |  |  |  |  |  |
| ton | Turn-on Time | IC= 1A ,RL= 20Ω, IB1= IB2= 0.1A,VCC= 20V |  | 0.26 |  | μs |
| tstg | Storage Time |  |  | 6.88 |  | μs |
| tf | Fall Time |  |  | 0.68 |  | μs |

- hFE-1 Classifications

| D | E |
| --- | --- |
| 60-120 | 100-200 |